

DIAGRAMATIC REPRESENTATION OF NEURAL NETWORK


[image: image1.png]OUTPUT LAYER

MIDDLE LAYER

INPUT LAYER





INTELLIGENT MANUFACTURING CONCEPT APPLIED TO INJECTION MOLDING PROCESS

                Injection molding makes a wide variety of plastic parts. When used with thermoplastics the process is basically consists of plastic compound, which isin powdered or granular form and then injecting the resultant melt into a mold either by a hydraulic plunger or by rotating screw system of an extruder. Injection molding of thermosets differs in the way, the melt is hardened in the mould.


[image: image2.png]/55N

))))
N )





APPLICATION OF NEURAL NETWORK BASED CONTROL TO INJECTION MOLDING PROCESS

The aim is to identify the feasibility of obtaining an intelligent control scheme that can learn the inverse model of a manufacturing process and react in a real-time to take corrective actions during fabrication.

The primary reasons for this are-

[image: image3.png]



Control of flow progression are, during the molding process required using the process model information in an inverse manner since there is no way of relating a desired flow progressionscheme to an inlet flow rate profile that will result in it and Injection molding process with two inlet gates are proposed during the present thesis.The steps that should be followed in obtaining the training and test data for the neural network proposed are:

[image: image4.png]



Control of flow progression during the molding process requires fast acting control unit since mold filled times are very short.

[image: image5.png]



Start with an empty mold and run the simulation code until the mold was filled, randomly changing the inlet flow rates every ten seconds.

[image: image6.png]



From the simulation, every two consecutive flow fronts plus the inlet flowrates that combine them constituted atraining (a test) pattern for the neural network.

FORWARD MOLDING WITH A BACK PROPAGATING NEURAL NETWORK

                   In the forward modeling case the neural network is given apresent flow front and an inlet flow rate combination and is expected to predict the shape and location of the next flow front

The control method can be summerised as follows:

[image: image7.png]



At any given time, sense the present flow front.

[image: image8.png]



Using a ‘C’ programme compare this present flow front with flow fronts along the desired flow progression part identified earlier and find the desired flow front that comes closest to and after the present flow front. This can be called the desired flow front of interest for the given time step.

[image: image9.png]



Run the neural network many times, each time with the input set to the neural network being the present flow front plus, a different flow rate combination.

[image: image10.png]



Compare the resulting predicted flow fronts obtained as neural networkoutputs with the desired flow fronts of interest and find the predicted flow rate combination that gave this predicted flow front is the best choice for this time step.

The four-step procedure is repeated until the mold cavity is filled.

CONTROL SYSTEM BY USING NEURAL NETWORKS


[image: image11.png]



CONTROL OF FLOW PROGRESSION DURING INJECTION MOLDING PROCESS

         The resulting flow front can then be fed into c programme, which can do the following operations:

It should find the desired flow front of interest.

· It should call the neural network 256 times (as specified earlier) compare each time with a different inlet flow rate combination plus the flow front obtained from the simulation software as the present flow front.

· It should compare the each of the resulting 256 possible next flow front with the desired flow front of interest.

· It should identify the possible next flow front that is closest to the desired flow front of interest.

CONCLUSIONS AND FUTURE DIRECTION

      On going efforts are directed towards demonstrating the capabilities of this process model/neural Network based control unit during the injection molding and resin transfer molding experiments that integrate the control method with a reliable flow progression sensing sub system . Once this is accomplished we would like to apply this novel process devolopment method to other manufacturing processes for which process models and in-situ schemes exists.

BIBLIOGRAPHY:

[image: image12.png]



Introductin to the artificial neural networks by “P.Yagna narayana”.

[image: image13.png]



ASME journals of manufacturing scheme &engineering.

































































































































































































www.Technicalsymposium.com


_1163922878

_1163923010

_1163922693

