Faculty Name		: Mr. A. SELVARAJ                 	Designation	: LECTURER

Department		: E.E.E.				Faculty Code	: EE 54

Subject Name		: M & I Laboratory             	Subject Code	: 131353

Year			: II	                             	Semester	: III

Degree & Branch	: B.E. – E.E.E.			Batch                   : A1

MEASUREMENTS AND INSTRUMENTATION LABORATORY –131353

LIST OF EXPERIMENTS

| S.NO | TITLE | PAGE NO |
| --- | --- | --- |
| 1 | MEASUREMENT OF RESISTANCE USING WHEATSTONE BRIDGE |  |
| 2 | MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE. |  |
| 3 | MEASUREMENT OF CAPACITANCE USING SCHERING BRIDGE. |  |
| 4 | MEASUREMENT OF INDUCTANCE USING MAXWELLS BRIDGE. |  |
| 5 | CALIBRATION OF 1 ENERGYMETER. |  |
| 6 | MEASUREMENT OF 3 PHASE POWER AND POWER FACTOR |  |
| 7 | INSTRUMENTATION AMPLIFIER. |  |
| 8 | STUDY OF DISPLACEMENT TRANSDUCER - LVDT |  |
| 9 | STUDY OF PRESSURE TRANSDUCER |  |
| 10 | A/D CONVERTER |  |
| 11 | D/A CONVERTER |  |
| 12 | STUDY OF TRANSIENTS |  |
| 13 | CALIBRATION OF CURRENT TRANSFORMER |  |
| 14 | MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE) |  |

EXPT. NO. 1	MEASUREMENT OF RESISTANCE USING

WHEATSTONE BRIDGE

# AIM:

To measure the given medium resistance using Wheatstone bridge.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Wheat stone Bridge kit Unknown resistance Multimeter Connecting Wires. |  |  |  |

CIRCUIT DIAGRAM:

# THEORY:

FORMULAE USED:

Unknown Resistance, Rx = (P/Q) S ()

Where   P, Q = Ratio Arms.

S = Variable resistance,

Rx = Unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected between XX points in the bridge kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing both the battery and the galvanometer keys.

3. The S arm (X 1000) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point and  kept at the lowest

value of S. Then the x100, x10, x1 knobs of S are adjusted to get null deflection.

If necessary the sensitivity knob may be adjusted to get appreciable deflection. [If not

possible P/Q ratio is kept at  any other suitable value ie, any one of ratios provided.]

4. The value of unknown resistance is read. (S value)

5. Steps 3 and 4 are repeated for some other P/Q ratio. The mean value is taken.

6. The experiment is repeated with other samples provided.

NOTE: The above experiment may be used for measuring resistance of the samples less than 1 to greater than 10k with lesser sensitivity.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (MULTIPLIER) | S VALUE () | UNKNOWN RESISTANCE RX () |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 2	MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE

# AIM:

To measure the given low resistance using Kelvin’s double bridge method.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to  study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Kelvin Double bridge kit Unknown resistance Multimeter Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

Rx = (P/Q)  S ohms

Where

P, Q  first set of ratio arms.

p, q   Second set of ratio arms.

S       Standard resistance,

Rx     unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected such that the leads from +C and + P are

connected to one end and those from –C and –P are connected to the other end in the

kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing the galvanometer key.

3. The ‘S’ arm (main dial) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point. [If not some other P/Q

ratio is to be tried].

4. The lowest of the two position indicates the coarse value of the unknown resistance

and the null point is obtained by adjusting the Vernier scale, with the galvanometer

sensitivity knob at the maximum position.

5. The value of unknown resistance is read. [‘S’ Value]

6. Steps 3, 4, 5 are repeated for some other P/Q ratio for the unknown resistance. The

mean value is taken.

7. The above procedure is repeated with another sample.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (Multiplier) | S VALUE COARSE() FINE ( ) | UNKNOWN RESISTANCE Rx () |
| --- | --- | --- | --- | --- |

.

CALCULATION:

RESULT:

EXPT. NO. 3	MEASUREMENT OF CAPACITANCE USING

SCHERING BRIDGE

# AIM:

To measure the unknown capacitance using Schering bridge.

# OBJECTIVE:

To measure the unknown capacitance and to study about dissipation factor.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 | Schering Bridge kit Unknown capacitance Multimeter Connecting Wires CRO |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

1.  Unknown capacitance, Cx = (R1/R2) C3,

Where   C3 is known capacitance in µF

R2 is Non-Inductive Variable Resistor

2. Dissipation factor, D = ω R1 C1 =  ω Rx Cx

PROCEDURE:

- The oscillator AB and the bridge AB terminals are connected.

- From the CD terminals of the bridge, the detector (loudspeaker) is connected through an imbalance amplifier.

- Connections of the bridge arm are made as per the circuit diagram.

- The value of R2 is selected arbitrarily (say1K) and R1 is kept at maximum position.

- The kit is switched on and R1 is decreased until the null point is observed as a dip in the sound from the loudspeaker.

- The capacitor C1 can be varied for fine balance adjustment.

- When the balance condition is reached, the trainer kit is switched OFF and the value R1 is measured using a multimeter.

- The value of unknown capacitance and dissipation factor is calculated.

- The experiment is repeated for various samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R2 () | R1 () | UNKNOWN CAPACITANCE Cx (µF) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 4 	MEASUREMENT OF INDUCTANCE USING

MAXWELLS BRIDGE

# AIM:

To find the unknown inductance and Q factor of a given coil.

# OBJECTIVE:

To find the unknown inductance of the given coil using bridge circuit and to study that Maxwell inductance- capacitance bridge is suitable for the measurement of low Q coils.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Maxwell’s inductance Capacitance Bridge kit Unknown Inductance Multimeter Connecting Wires CRO Bridge oscillator | 1 kHZ |  |  |

CIRCUIT DIAGRAM:

R3

L1

R1

D

C4

R2

R4

# THEORY:

# FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

Thus we have two variables R4 and C4 which appears in one of the two balance equation and hence the two equations are independent, and balance is obtained by varying R4 and C4 alternately.

3. Quality factor, Q = ωL1 / R1 = ωC4R4

PROCEDURE:

- The inductance to be measured is connected between L1 terminal of the kit.

- The bridge oscillator is set for 10Vpp, 1 kHz and connected to the OSC terminal of the kit.

- The detector CRO or headphone is connected to the headphone terminal of the kit.

- The R4 and C4 are adjusted from the highest range (among the 3 range knobs) to obtain the null point in the detector. [ Null point – For increase in R4 and C4 values the point at which the amplitude reduces to a minimum and then increases is null point).

- At the null point the values of R4 and C4 are noted.

- The value of unknown resistance, inductance and quality factor are calculated.

- The experiment is repeated with other samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R4(Ω) | C4 (F) | UNKNOWN INDUCTANCE (H) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 5	CALIBRATION OF SINGLE PHASE ENERGYMETER

AIM:

To calibrate the given single phase energy meter at unity and other power factors

OBJECTIVE:

- To study the working of energy meter.

- To accurately calibrate the meter at unity and other power factor.

- To study the % of error for the given energy meter.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Energy meter Wattmeter Ammeter Voltmeter Stop watch Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

NAME PLATE DETAILS:

| RATED CURRENT |  |
| --- | --- |
| RATED VOLTAGE |  |
| FREQUENCY |  |
| REVOLUTIONS/KWH |  |

THEORY:

FORMULAE USED:

Let x revolution / kwh be the rating.

Now x revolution = 1 kwh

= 1* 3600*1000 watt-sec.

Constant k of energymeter  = 3600 * 103/ x watt-sec

For each load, indicated power Wi  is given as Wi = k/t watts

Where

K= energy meter constant (watt-sec)

t = time for 1 revolution(sec)

% error = Wi – Wa / Wi * 100

Where Wi  is indicated power in watts

Wa is actual power shown by wattmeter in watts

% error can be zero +ve or –ve.

PROCEDURE:

1. Connections are given as shown in the circuit diagram.

2. Supply is switched ON and load is increased in steps, each time noting the readings of

ammeter and wattmeter. Also the actual time taken for 1 revolution of the disc is

measured using stop watch.

3. Step 2 is repeated till rated current of the energy meter is reached.

4. % error is calculated and calibration curve is drawn.

TABULAR COLUMN:

| S.NO | LOAD CURRENT I (Amps) | WATTMETER READING, Wa (W) | INDICATED POWER, Wi (W) | Time taken , t (secs) | % ERROR |
| --- | --- | --- | --- | --- | --- |

CALCULATION:

MODEL GRAPH:

+Ve

% Error

0

Wi

-Ve

NOTE:

From the calibration curve it is possible to predict the error in recording the    energy. So the correction can be applied to the energy meter reading so that correct energy reading can be obtained and used.

RESULT:

EXPT. NO. 6        MEASUREMENT OF THREE PHASE POWER

AND POWER FACTOR

AIM

To conduct a suitable experiment on a 3-phase load connected in star or delta to measure the three phase power and power factor using 2 wattmeter method.

OBJECTIVES

- To study the working of wattmeter

- To accurately measure the 3 phase power

- To accurately measure the power factor

- To study the concept of star connected load and delta connected load

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Ammeter Voltmeter Wattmeter Connecting Wires 3 phase inductive load |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

- Total power, P = W1+W2 (W)

- = Tan-1 3 [(W1-W2) /(W1+W2)]

3.   P.F = Cos

PROCEDURE:

- Connection are made as per the circuit diagram, keeping the inductive load in the

initial position.

- Supply switch is closed and reading of ammeter and wattmeter are noted .If one of the    wattmeter reads negative, then its potential coils (C and V) are interchanged and

reading is taken as negative.

3.  The above procedure is repeated for different values of inductive coil. Care should be

taken that current should not exceed 10A during the experiment.

TABULAR COLUMN:

| S.NO | I | W1 | W2 | POWER, P | P.F Cos |
| --- | --- | --- | --- | --- | --- |
|  | (Amps) | (Watts) | (Watts) | (Watts) |  |

CALCULATION:

RESULT:

EXPT. NO. 7	INSTRUMENTATION AMPLIFIER.

AIM:

To study the working of instrumentation amplifier.

OBJECTIVE:

- To study the characteristic of operational amplifier.

- To study the use of operational amplifier as instrumentation amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Instrumentation Amplifier Kit Dual RPS Multi meter Connecting Wires |  |  |

PIN DIAGRAM:

PIN DIAGRAM OF IC741

CIRCUIT DIAGRAM:

R1 = R2 = R

THEORY:

FORMULAE USED:

- Vo= (V2 – V1) (1 + 2R / Rg)

PROCEDURE:

- Connect a dual RPS to the input terminals V1 and V2 of either channel 1 or channel 2 of instrumentation amplifier module.

- Adjust the RPS for different voltages for V1 and V2 (not more than 10 V).

- Measure the output voltage using a multi meter for every set of V1 and V2.

TABULAR COLUMN:

| V1 (v) | V2 (v) | Vout (v) |
| --- | --- | --- |

RESULT:

EXPT. NO. 8 	STUDY OF DISPLACEMENT TRANSDUCER - LVDT

# AIM

To study the operation of LVDT- Linear Variable Differential Transformer

# OBJECTIVE:

- To study the basic principle of LVDT.

- Study of signal conditioning circuit.

3.         Study of LVDT as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | LVDT kit Multimeter Connecting Wires |  |  |

THEORY:

CIRCUIT DIAGRAM:

Fig.1

Fig.2

PATCHING DIAGRAM:

PROCEDURE:

- The trainer is switched and power supply is checked for +12V and -12Vwith the help of multi meter.

- The trainer is switched off and connections are made as per the patching diagram.

- The LVDT is connected in the kit through connector provided in the circuit.

- The displacement of LVDT is kept at one end by  using screw guage.

- The trainer is switched on and the output DC voltage is measured using multimeter by connecting it across DC output + and – respectively.

- The displacement is varied with the help of screw gauge and the corresponding output voltages are noted down.

- The screw gauge is adjusted for maximum displacement and then stopped.

- A graph between displacement and output voltage is drawn.

TABULAR COLUMN:

| S.NO | DISPLACEMENT ON MICRO-METER (mm) | MULTIMETER READING (Volts) |
| --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 9                          STUDY OF PRESSURE TRANSDUCER

# AIM

To study the operation of a pressure transducer – Strain gauge.

# OBJECTIVES

- To study the basic principle of   Strain gauge.

- Study of Strain gauge as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 | Pressure measurement module Multimeter |  |  |  |

THEORY:

FORMULAE USED:

% Error = [(Gauge pressure – displayed pressure) / displayed pressure] * 100

PROCEDURE:

- Install the pressure cell setup and interface the 9 pin D connector with the pressure measurement module.  Switch ON the module.

- Calibrate the pressure measurement module   by   the following procedure. Open  the air release valve and exhaust the tank inlet air and nullify the output voltage by using zero adjustment pot, then close the air release valve and apply 20 Psi  pressure to the cylinder and adjust the display to 20 Psi by using gain adjustment POT.

- After calibration open the air release valve and exhaust the tank inlet air.

- Close the air release valve and press the pump piston and note down the bridge voltage in mV across T2 and T3, output   voltage   in V across T5 and GND and the displayed pressure in Psi in the indicator.  Repeat the procedure for increasing values of pressure and tabulate the readings.(Pressure should not exceed 20 Psi)

- Plot the error calibration curve.

CIRCUIT DIAGRAM:

TABULAR COLUMN:

| S. No. | Gauge Pressure (Psi) | Displayed Pressure (Psi) | Bridge Voltage (mV) | Output Voltage (V) | %Error |
| --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 10			A/D CONVERTER

AIM

To study the working of successive approximation type Analog to Digital converter.

OBJECTIVE

- To study the conversion of analog i/p voltage to digital o/p voltage.

- To study the operation and characteristics of operational amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | A/D Converter kit RPS Multimeter Connecting Wires |  |  |  |

THEORY:

FORMULAE USED:

Input voltage, Vin = Vref [D7 2-1 + D6 2-2 + D5 2-3 + ………. + D0 2-8]

Where,

Vref = Reference voltage

D7, D6,D5,……D0 = 8 bit digital output

D7  - MSB of digital data

D0 – LSB of digital data

CIRCUIT DIAGRAM:

TABULAR COLUMN:

Reference Voltage –

SW1 =0			SW2 = 1				SW3=1

| Analog input voltage |  | Digital output |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theoretical | Practical | D7 | D6 |  | D5 | D4 |  | D3 | D2 |  | D1 | D0 |

PROCEDURE:

- The channel 3 is selected by using switches SW1, SW2, SW3 by keeping them in 011 position.

- Vref is measured using multi meter across pin 6 and 1 by keeping the pot at maximum position.

- The analog voltage to be converted is set by using the pot and the voltage set is measured by using multi meter.

- The ALE switch SW5 is pressed once to send the data to SAR.

- The SOC switch SW4 is pressed once to obtain the digital data for the given input voltage. The digital output obtained is noted.

- The procedure is repeated from step 2 for various values of analog input voltage.

RESULT:

EXPT. NO. 11			D/A CONVERTER

AIM

To study the working of an 8 bit weighted resistor type Digital to Analog converter.

OBJECTIVE

- To study the conversion of binary voltage to analog o/p voltage

- To study the operation and characteristics of operational amplifier

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | D/A Converter kit RPS Multimeter Connecting Wires |  |  |

THEORY:

FORMULAE USED:

Eo = Vref [(2 Xo – 255) / 256]

Where Xo is the equivalent decimal value of the given digital output.

CIRCUIT DIAGRAM:

PROCEDURE:

- The connections are made as per the circuit diagram.

- Vref is measured across P2 terminal by keeping the pot knob in maximum position.

- The binary input is applied by closing the appropriate switches.

- The analog output across P2 terminal by using multi meter.

- The procedure is repeated for various binary input values.

TABULAR COLUMN:

| S.No. | Digital Input |  | Analog output |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Binary | Decimal equivalent | Theoretical |  | Practical |

RESULT:

EXPT.NO. 12				STUDY OF TRANSIENTS

- RC TRANSIENTS:

AIM

To Observe and plot the transients waveform for a series RC circuit and compute the time constant.

OBJECTIVE

- To study the transient behaviour of the given system

- To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Capacitance Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4τ -1 where τ =RC

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR = Ve- t/RC is observed in the CRO and the time taken by the output to reach 36.8% of its final value (discharging time) is noted.

MODEL GRAPH:

RESULT:

- RL TRANSIENTS:

AIM:

To observe and plot the transient waveform for a series RL circuit and to compute the time constant

OBJECTIVE

1. To study the transient behavior of the given system

2. To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Inductor Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4 τ)-1 where τ =L/R

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR =V ( 1 - e- tR/L) is observed in the CRO and the time taken by the output to reach 63.2% of its final value (charging time) is noted.

MODEL GRAPH:

RESULT:

EXPT. NO. 13	CALIBRATION OF CURRENT TRANSFORMER

AIM

To study the working of current transformer

OBJECTIVE

- To study the current transformation concept

- To study the efficiency of a given current transformer

- To study the loss components in the circuit

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Current Transformer Voltmeter Ammeter Resistive Load |  |  |

THEORY:

FORMULAE USED:

% Ratio error =[( nominal ratio – actual ratio) / actual ratio] x 100

= [ (Kn  - Kact) / Kact ]

Where

Kact  = magnitude of actual primary current / magnitude of actual  secondary current

Kn  = rated primary current / rated secondary current

Inductive reactance in Ω, XL = 2πfL

Where L is inductance of CT secondary in H

Impedance in Ω, Z = √ R2 + XL2

Secondary burden due to load, VA = I2 Z

PRECAUTIONS:

- SPST 1 and SPST 2 should be kept closed before switching ON the supply.

- SPST2 should be opened before applying the load.

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- DPST is closed on the primary side and no load voltage and current are noted.

- SPST 2 is opened and then load is applied in steps. For each load applied, line voltage, primary current, secondary current, and load values are noted down. Also resistance and inductance values of CT secondary are measured using LCR meter.

- The % error and secondary burden due to lad are calculated.

- A graph is drawn between primary current and secondary current is drawn.

TABULAR COLUMN:

| S. No | Line voltage V | Primary current A | Secondary current A | Load W | R Ω | L H | Z Ω | % Error % | Secondary burden due to load VA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO. 14	MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE)

AIM

To measure the iron loss of a given ring specimen

OBJECTIVE

- To study about hysterisis loss

- To study about eddy current loss

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Measurement of iron loss trainer Ring Specimen Multimeter CRO |  |  |

THEORY:

FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

3.  Iron loss = I12 x (R1 – Rw)

Where,

Rw is the winding resistance of the  ring specimen measured using  multimeter.

CIRCUIT DIAGRAM:

PROCEDURE:

- Patch the circuit as per the circuit diagram

- The pot 2(R4) is kept in the maximum position and R3 is selected as 10 Ω initially.

- The trainer kit is switched ON

- Adjust pot 1 (R2) and obtain the null position in the CRO.

- At  the  bridge balance condition  note down the current I1 using multi meter(in mA range selection) connected in series with R3  .

- Switch OFF the trainer kit and note down the value of R2 and winding resistance of ring specimen using multi meter.

- Steps 3 to 6 are repeated for other two settings of R3.

- The entire procedure is repeated for other samples of ring specimens provided in the kit.

TABULAR COLUMN:					R4  = __________ (Ω)

| S.No. | Specimen No. | R2 (Ω) | R3(Ω) | I1 (mA) | RW(Ω) | R1(Ω) | L1(H) | Iron loss (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | 1 |  | 10 |  |  |  |  |  |
| 2. |  |  | 100 |  |  |  |  |  |
| 3. |  |  | 1000 |  |  |  |  |  |
| 4. | 2 |  | 10 |  |  |  |  |  |
| 5. |  |  | 100 |  |  |  |  |  |
| 6. |  |  | 1000 |  |  |  |  |  |
| 7. | 3 |  | 10 |  |  |  |  |  |
| 8. |  |  | 100 |  |  |  |  |  |
| 9. |  |  | 1000 |  |  |  |  |  |

RESULT:

Faculty Name		: Mrs. V. DEVI MAHESWARAN      Designation:     ASSISTANT PROFESSOR

Department		: E.E.E.				Faculty Code	: EE48

Subject Name		: M & I Laboratory             	Subject Code	: 131353

Year			: II	                             	Semester	: III

Degree & Branch	: B.E. – E.E.E.			Batch                   : A2

MEASUREMENTS AND INSTRUMENTATION LABORATORY –131353

LIST OF EXPERIMENTS

| S.NO | TITLE | PAGE NO |
| --- | --- | --- |
| 1 | MEASUREMENT OF RESISTANCE USING WHEATSTONE BRIDGE |  |
| 2 | MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE. |  |
| 3 | MEASUREMENT OF CAPACITANCE USING SCHERING BRIDGE. |  |
| 4 | MEASUREMENT OF INDUCTANCE USING MAXWELLS BRIDGE. |  |
| 5 | CALIBRATION OF 1 ENERGYMETER. |  |
| 6 | MEASUREMENT OF 3 PHASE POWER AND POWER FACTOR |  |
| 7 | INSTRUMENTATION AMPLIFIER. |  |
| 8 | STUDY OF DISPLACEMENT TRANSDUCER - LVDT |  |
| 9 | STUDY OF PRESSURE TRANSDUCER |  |
| 10 | A/D CONVERTER |  |
| 11 | D/A CONVERTER |  |
| 12 | STUDY OF TRANSIENTS |  |
| 13 | CALIBRATION OF CURRENT TRANSFORMER |  |
| 14 | MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE) |  |

EXPT. NO. 1	MEASUREMENT OF RESISTANCE USING

WHEATSTONE BRIDGE

# AIM:

To measure the given medium resistance using Wheatstone bridge.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Wheat stone Bridge kit Unknown resistance Multimeter Connecting Wires. |  |  |  |

CIRCUIT DIAGRAM:

# THEORY:

FORMULAE USED:

Unknown Resistance, Rx = (P/Q) S ()

Where   P, Q = Ratio Arms.

S = Variable resistance,

Rx = Unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected between XX points in the bridge kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing both the battery and the galvanometer keys.

3. The S arm (X 1000) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point and  kept at the lowest

value of S. Then the x100, x10, x1 knobs of S are adjusted to get null deflection.

If necessary the sensitivity knob may be adjusted to get appreciable deflection. [If not

possible P/Q ratio is kept at  any other suitable value ie, any one of ratios provided.]

4. The value of unknown resistance is read. (S value)

5. Steps 3 and 4 are repeated for some other P/Q ratio. The mean value is taken.

6. The experiment is repeated with other samples provided.

NOTE: The above experiment may be used for measuring resistance of the samples less than 1 to greater than 10k with lesser sensitivity.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (MULTIPLIER) | S VALUE () | UNKNOWN RESISTANCE RX () |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 2	MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE

# AIM:

To measure the given low resistance using Kelvin’s double bridge method.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to  study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Kelvin Double bridge kit Unknown resistance Multimeter Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

Rx = (P/Q)  S ohms

Where

P, Q  first set of ratio arms.

p, q   Second set of ratio arms.

S       Standard resistance,

Rx     unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected such that the leads from +C and + P are

connected to one end and those from –C and –P are connected to the other end in the

kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing the galvanometer key.

3. The ‘S’ arm (main dial) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point. [If not some other P/Q

ratio is to be tried].

4. The lowest of the two position indicates the coarse value of the unknown resistance

and the null point is obtained by adjusting the Vernier scale, with the galvanometer

sensitivity knob at the maximum position.

5. The value of unknown resistance is read. [‘S’ Value]

6. Steps 3, 4, 5 are repeated for some other P/Q ratio for the unknown resistance. The

mean value is taken.

7. The above procedure is repeated with another sample.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (Multiplier) | S VALUE COARSE() FINE ( ) | UNKNOWN RESISTANCE Rx () |
| --- | --- | --- | --- | --- |

.

CALCULATION:

RESULT:

EXPT. NO. 3	MEASUREMENT OF CAPACITANCE USING

SCHERING BRIDGE

# AIM:

To measure the unknown capacitance using Schering bridge.

# OBJECTIVE:

To measure the unknown capacitance and to study about dissipation factor.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 | Schering Bridge kit Unknown capacitance Multimeter Connecting Wires CRO |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

1.  Unknown capacitance, Cx = (R1/R2) C3,

Where   C3 is known capacitance in µF

R2 is Non-Inductive Variable Resistor

2. Dissipation factor, D = ω R1 C1 =  ω Rx Cx

PROCEDURE:

- The oscillator AB and the bridge AB terminals are connected.

- From the CD terminals of the bridge, the detector (loudspeaker) is connected through an imbalance amplifier.

- Connections of the bridge arm are made as per the circuit diagram.

- The value of R2 is selected arbitrarily (say1K) and R1 is kept at maximum position.

- The kit is switched on and R1 is decreased until the null point is observed as a dip in the sound from the loudspeaker.

- The capacitor C1 can be varied for fine balance adjustment.

- When the balance condition is reached, the trainer kit is switched OFF and the value R1 is measured using a multimeter.

- The value of unknown capacitance and dissipation factor is calculated.

- The experiment is repeated for various samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R2 () | R1 () | UNKNOWN CAPACITANCE Cx (µF) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 4 	MEASUREMENT OF INDUCTANCE USING

MAXWELLS BRIDGE

# AIM:

To find the unknown inductance and Q factor of a given coil.

# OBJECTIVE:

To find the unknown inductance of the given coil using bridge circuit and to study that Maxwell inductance- capacitance bridge is suitable for the measurement of low Q coils.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Maxwell’s inductance Capacitance Bridge kit Unknown Inductance Multimeter Connecting Wires CRO Bridge oscillator | 1 kHZ |  |  |

CIRCUIT DIAGRAM:

R3

L1

R1

D

C4

R2

R4

# THEORY:

# FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

Thus we have two variables R4 and C4 which appears in one of the two balance equation and hence the two equations are independent, and balance is obtained by varying R4 and C4 alternately.

3. Quality factor, Q = ωL1 / R1 = ωC4R4

PROCEDURE:

- The inductance to be measured is connected between L1 terminal of the kit.

- The bridge oscillator is set for 10Vpp, 1 kHz and connected to the OSC terminal of the kit.

- The detector CRO or headphone is connected to the headphone terminal of the kit.

- The R4 and C4 are adjusted from the highest range (among the 3 range knobs) to obtain the null point in the detector. [ Null point – For increase in R4 and C4 values the point at which the amplitude reduces to a minimum and then increases is null point).

- At the null point the values of R4 and C4 are noted.

- The value of unknown resistance, inductance and quality factor are calculated.

- The experiment is repeated with other samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R4(Ω) | C4 (F) | UNKNOWN INDUCTANCE (H) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 5	CALIBRATION OF SINGLE PHASE ENERGYMETER

AIM:

To calibrate the given single phase energy meter at unity and other power factors

OBJECTIVE:

- To study the working of energy meter.

- To accurately calibrate the meter at unity and other power factor.

- To study the % of error for the given energy meter.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Energy meter Wattmeter Ammeter Voltmeter Stop watch Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

NAME PLATE DETAILS:

| RATED CURRENT |  |
| --- | --- |
| RATED VOLTAGE |  |
| FREQUENCY |  |
| REVOLUTIONS/KWH |  |

THEORY:

FORMULAE USED:

Let x revolution / kwh be the rating.

Now x revolution = 1 kwh

= 1* 3600*1000 watt-sec.

Constant k of energymeter  = 3600 * 103/ x watt-sec

For each load, indicated power Wi  is given as Wi = k/t watts

Where

K= energy meter constant (watt-sec)

t = time for 1 revolution(sec)

% error = Wi – Wa / Wi * 100

Where Wi  is indicated power in watts

Wa is actual power shown by wattmeter in watts

% error can be zero +ve or –ve.

PROCEDURE:

1. Connections are given as shown in the circuit diagram.

2. Supply is switched ON and load is increased in steps, each time noting the readings of

ammeter and wattmeter. Also the actual time taken for 1 revolution of the disc is

measured using stop watch.

3. Step 2 is repeated till rated current of the energy meter is reached.

4. % error is calculated and calibration curve is drawn.

TABULAR COLUMN:

| S.NO | LOAD CURRENT I (Amps) | WATTMETER READING, Wa (W) | INDICATED POWER, Wi (W) | Time taken , t (secs) | % ERROR |
| --- | --- | --- | --- | --- | --- |

CALCULATION:

MODEL GRAPH:

+Ve

% Error

0

Wi

-Ve

NOTE:

From the calibration curve it is possible to predict the error in recording the    energy. So the correction can be applied to the energy meter reading so that correct energy reading can be obtained and used.

RESULT:

EXPT. NO. 6        MEASUREMENT OF THREE PHASE POWER

AND POWER FACTOR

AIM

To conduct a suitable experiment on a 3-phase load connected in star or delta to measure the three phase power and power factor using 2 wattmeter method.

OBJECTIVES

- To study the working of wattmeter

- To accurately measure the 3 phase power

- To accurately measure the power factor

- To study the concept of star connected load and delta connected load

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Ammeter Voltmeter Wattmeter Connecting Wires 3 phase inductive load |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

- Total power, P = W1+W2 (W)

- = Tan-1 3 [(W1-W2) /(W1+W2)]

3.   P.F = Cos

PROCEDURE:

- Connection are made as per the circuit diagram, keeping the inductive load in the

initial position.

- Supply switch is closed and reading of ammeter and wattmeter are noted .If one of the    wattmeter reads negative, then its potential coils (C and V) are interchanged and

reading is taken as negative.

3.  The above procedure is repeated for different values of inductive coil. Care should be

taken that current should not exceed 10A during the experiment.

TABULAR COLUMN:

| S.NO | I | W1 | W2 | POWER, P | P.F Cos |
| --- | --- | --- | --- | --- | --- |
|  | (Amps) | (Watts) | (Watts) | (Watts) |  |

CALCULATION:

RESULT:

EXPT. NO. 7	INSTRUMENTATION AMPLIFIER.

AIM:

To study the working of instrumentation amplifier.

OBJECTIVE:

- To study the characteristic of operational amplifier.

- To study the use of operational amplifier as instrumentation amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Instrumentation Amplifier Kit Dual RPS Multi meter Connecting Wires |  |  |

PIN DIAGRAM:

PIN DIAGRAM OF IC741

CIRCUIT DIAGRAM:

R1 = R2 = R

THEORY:

FORMULAE USED:

- Vo= (V2 – V1) (1 + 2R / Rg)

PROCEDURE:

- Connect a dual RPS to the input terminals V1 and V2 of either channel 1 or channel 2 of instrumentation amplifier module.

- Adjust the RPS for different voltages for V1 and V2 (not more than 10 V).

- Measure the output voltage using a multi meter for every set of V1 and V2.

TABULAR COLUMN:

| V1 (v) | V2 (v) | Vout (v) |
| --- | --- | --- |

RESULT:

EXPT. NO. 8 	STUDY OF DISPLACEMENT TRANSDUCER - LVDT

# AIM

To study the operation of LVDT- Linear Variable Differential Transformer

# OBJECTIVE:

- To study the basic principle of LVDT.

- Study of signal conditioning circuit.

3.         Study of LVDT as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | LVDT kit Multimeter Connecting Wires |  |  |

THEORY:

CIRCUIT DIAGRAM:

Fig.1

Fig.2

PATCHING DIAGRAM:

PROCEDURE:

- The trainer is switched and power supply is checked for +12V and -12Vwith the help of multi meter.

- The trainer is switched off and connections are made as per the patching diagram.

- The LVDT is connected in the kit through connector provided in the circuit.

- The displacement of LVDT is kept at one end by  using screw guage.

- The trainer is switched on and the output DC voltage is measured using multimeter by connecting it across DC output + and – respectively.

- The displacement is varied with the help of screw gauge and the corresponding output voltages are noted down.

- The screw gauge is adjusted for maximum displacement and then stopped.

- A graph between displacement and output voltage is drawn.

TABULAR COLUMN:

| S.NO | DISPLACEMENT ON MICRO-METER (mm) | MULTIMETER READING (Volts) |
| --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 9                          STUDY OF PRESSURE TRANSDUCER

# AIM

To study the operation of a pressure transducer – Strain gauge.

# OBJECTIVES

- To study the basic principle of   Strain gauge.

- Study of Strain gauge as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 | Pressure measurement module Multimeter |  |  |  |

THEORY:

FORMULAE USED:

% Error = [(Gauge pressure – displayed pressure) / displayed pressure] * 100

PROCEDURE:

- Install the pressure cell setup and interface the 9 pin D connector with the pressure measurement module.  Switch ON the module.

- Calibrate the pressure measurement module   by   the following procedure. Open  the air release valve and exhaust the tank inlet air and nullify the output voltage by using zero adjustment pot, then close the air release valve and apply 20 Psi  pressure to the cylinder and adjust the display to 20 Psi by using gain adjustment POT.

- After calibration open the air release valve and exhaust the tank inlet air.

- Close the air release valve and press the pump piston and note down the bridge voltage in mV across T2 and T3, output   voltage   in V across T5 and GND and the displayed pressure in Psi in the indicator.  Repeat the procedure for increasing values of pressure and tabulate the readings.(Pressure should not exceed 20 Psi)

- Plot the error calibration curve.

CIRCUIT DIAGRAM:

TABULAR COLUMN:

| S. No. | Gauge Pressure (Psi) | Displayed Pressure (Psi) | Bridge Voltage (mV) | Output Voltage (V) | %Error |
| --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 10			A/D CONVERTER

AIM

To study the working of successive approximation type Analog to Digital converter.

OBJECTIVE

- To study the conversion of analog i/p voltage to digital o/p voltage.

- To study the operation and characteristics of operational amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | A/D Converter kit RPS Multimeter Connecting Wires |  |  |  |

THEORY:

FORMULAE USED:

Input voltage, Vin = Vref [D7 2-1 + D6 2-2 + D5 2-3 + ………. + D0 2-8]

Where,

Vref = Reference voltage

D7, D6,D5,……D0 = 8 bit digital output

D7  - MSB of digital data

D0 – LSB of digital data

CIRCUIT DIAGRAM:

TABULAR COLUMN:

Reference Voltage –

SW1 =0			SW2 = 1				SW3=1

| Analog input voltage |  | Digital output |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theoretical | Practical | D7 | D6 |  | D5 | D4 |  | D3 | D2 |  | D1 | D0 |

PROCEDURE:

- The channel 3 is selected by using switches SW1, SW2, SW3 by keeping them in 011 position.

- Vref is measured using multi meter across pin 6 and 1 by keeping the pot at maximum position.

- The analog voltage to be converted is set by using the pot and the voltage set is measured by using multi meter.

- The ALE switch SW5 is pressed once to send the data to SAR.

- The SOC switch SW4 is pressed once to obtain the digital data for the given input voltage. The digital output obtained is noted.

- The procedure is repeated from step 2 for various values of analog input voltage.

RESULT:

EXPT. NO. 11			D/A CONVERTER

AIM

To study the working of an 8 bit weighted resistor type Digital to Analog converter.

OBJECTIVE

- To study the conversion of binary voltage to analog o/p voltage

- To study the operation and characteristics of operational amplifier

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | D/A Converter kit RPS Multimeter Connecting Wires |  |  |

THEORY:

FORMULAE USED:

Eo = Vref [(2 Xo – 255) / 256]

Where Xo is the equivalent decimal value of the given digital output.

CIRCUIT DIAGRAM:

PROCEDURE:

- The connections are made as per the circuit diagram.

- Vref is measured across P2 terminal by keeping the pot knob in maximum position.

- The binary input is applied by closing the appropriate switches.

- The analog output across P2 terminal by using multi meter.

- The procedure is repeated for various binary input values.

TABULAR COLUMN:

| S.No. | Digital Input |  | Analog output |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Binary | Decimal equivalent | Theoretical |  | Practical |

RESULT:

EXPT.NO. 12				STUDY OF TRANSIENTS

- RC TRANSIENTS:

AIM

To Observe and plot the transients waveform for a series RC circuit and compute the time constant.

OBJECTIVE

- To study the transient behaviour of the given system

- To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Capacitance Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4τ -1 where τ =RC

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR = Ve- t/RC is observed in the CRO and the time taken by the output to reach 36.8% of its final value (discharging time) is noted.

MODEL GRAPH:

RESULT:

- RL TRANSIENTS:

AIM:

To observe and plot the transient waveform for a series RL circuit and to compute the time constant

OBJECTIVE

1. To study the transient behavior of the given system

2. To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Inductor Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4 τ)-1 where τ =L/R

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR =V ( 1 - e- tR/L) is observed in the CRO and the time taken by the output to reach 63.2% of its final value (charging time) is noted.

MODEL GRAPH:

RESULT:

EXPT. NO. 13	CALIBRATION OF CURRENT TRANSFORMER

AIM

To study the working of current transformer

OBJECTIVE

- To study the current transformation concept

- To study the efficiency of a given current transformer

- To study the loss components in the circuit

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Current Transformer Voltmeter Ammeter Resistive Load |  |  |

THEORY:

FORMULAE USED:

% Ratio error =[( nominal ratio – actual ratio) / actual ratio] x 100

= [ (Kn  - Kact) / Kact ]

Where

Kact  = magnitude of actual primary current / magnitude of actual  secondary current

Kn  = rated primary current / rated secondary current

Inductive reactance in Ω, XL = 2πfL

Where L is inductance of CT secondary in H

Impedance in Ω, Z = √ R2 + XL2

Secondary burden due to load, VA = I2 Z

PRECAUTIONS:

- SPST 1 and SPST 2 should be kept closed before switching ON the supply.

- SPST2 should be opened before applying the load.

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- DPST is closed on the primary side and no load voltage and current are noted.

- SPST 2 is opened and then load is applied in steps. For each load applied, line voltage, primary current, secondary current, and load values are noted down. Also resistance and inductance values of CT secondary are measured using LCR meter.

- The % error and secondary burden due to lad are calculated.

- A graph is drawn between primary current and secondary current is drawn.

TABULAR COLUMN:

| S. No | Line voltage V | Primary current A | Secondary current A | Load W | R Ω | L H | Z Ω | % Error % | Secondary burden due to load VA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO. 14	MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE)

AIM

To measure the iron loss of a given ring specimen

OBJECTIVE

- To study about hysterisis loss

- To study about eddy current loss

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Measurement of iron loss trainer Ring Specimen Multimeter CRO |  |  |

THEORY:

FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

3.  Iron loss = I12 x (R1 – Rw)

Where,

Rw is the winding resistance of the  ring specimen measured using  multimeter.

CIRCUIT DIAGRAM:

PROCEDURE:

- Patch the circuit as per the circuit diagram

- The pot 2(R4) is kept in the maximum position and R3 is selected as 10 Ω initially.

- The trainer kit is switched ON

- Adjust pot 1 (R2) and obtain the null position in the CRO.

- At  the  bridge balance condition  note down the current I1 using multi meter(in mA range selection) connected in series with R3  .

- Switch OFF the trainer kit and note down the value of R2 and winding resistance of ring specimen using multi meter.

- Steps 3 to 6 are repeated for other two settings of R3.

- The entire procedure is repeated for other samples of ring specimens provided in the kit.

TABULAR COLUMN:					R4  = __________ (Ω)

| S.No. | Specimen No. | R2 (Ω) | R3(Ω) | I1 (mA) | RW(Ω) | R1(Ω) | L1(H) | Iron loss (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | 1 |  | 10 |  |  |  |  |  |
| 2. |  |  | 100 |  |  |  |  |  |
| 3. |  |  | 1000 |  |  |  |  |  |
| 4. | 2 |  | 10 |  |  |  |  |  |
| 5. |  |  | 100 |  |  |  |  |  |
| 6. |  |  | 1000 |  |  |  |  |  |
| 7. | 3 |  | 10 |  |  |  |  |  |
| 8. |  |  | 100 |  |  |  |  |  |
| 9. |  |  | 1000 |  |  |  |  |  |

RESULT:

Faculty Name		: Mrs. P.T. SUBASINI                     Designation:     LECTURER

Department		: E.E.E.				Faculty Code	: EE 41

Subject Name		: M & I Laboratory             	Subject Code	: 131353

Year			: II	                             	Semester	: III

Degree & Branch	: B.E. – E.E.E.			Batch                   :  B1

MEASUREMENTS AND INSTRUMENTATION LABORATORY –131353

LIST OF EXPERIMENTS

| S.NO | TITLE | PAGE NO |
| --- | --- | --- |
| 1 | MEASUREMENT OF RESISTANCE USING WHEATSTONE BRIDGE |  |
| 2 | MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE. |  |
| 3 | MEASUREMENT OF CAPACITANCE USING SCHERING BRIDGE. |  |
| 4 | MEASUREMENT OF INDUCTANCE USING MAXWELLS BRIDGE. |  |
| 5 | CALIBRATION OF 1 ENERGYMETER. |  |
| 6 | MEASUREMENT OF 3 PHASE POWER AND POWER FACTOR |  |
| 7 | INSTRUMENTATION AMPLIFIER. |  |
| 8 | STUDY OF DISPLACEMENT TRANSDUCER - LVDT |  |
| 9 | STUDY OF PRESSURE TRANSDUCER |  |
| 10 | A/D CONVERTER |  |
| 11 | D/A CONVERTER |  |
| 12 | STUDY OF TRANSIENTS |  |
| 13 | CALIBRATION OF CURRENT TRANSFORMER |  |
| 14 | MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE) |  |

EXPT. NO. 1	MEASUREMENT OF RESISTANCE USING

WHEATSTONE BRIDGE

# AIM:

To measure the given medium resistance using Wheatstone bridge.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Wheat stone Bridge kit Unknown resistance Multimeter Connecting Wires. |  |  |  |

CIRCUIT DIAGRAM:

# THEORY:

FORMULAE USED:

Unknown Resistance, Rx = (P/Q) S ()

Where   P, Q = Ratio Arms.

S = Variable resistance,

Rx = Unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected between XX points in the bridge kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing both the battery and the galvanometer keys.

3. The S arm (X 1000) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point and  kept at the lowest

value of S. Then the x100, x10, x1 knobs of S are adjusted to get null deflection.

If necessary the sensitivity knob may be adjusted to get appreciable deflection. [If not

possible P/Q ratio is kept at  any other suitable value ie, any one of ratios provided.]

4. The value of unknown resistance is read. (S value)

5. Steps 3 and 4 are repeated for some other P/Q ratio. The mean value is taken.

6. The experiment is repeated with other samples provided.

NOTE: The above experiment may be used for measuring resistance of the samples less than 1 to greater than 10k with lesser sensitivity.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (MULTIPLIER) | S VALUE () | UNKNOWN RESISTANCE RX () |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 2	MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE

# AIM:

To measure the given low resistance using Kelvin’s double bridge method.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to  study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Kelvin Double bridge kit Unknown resistance Multimeter Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

Rx = (P/Q)  S ohms

Where

P, Q  first set of ratio arms.

p, q   Second set of ratio arms.

S       Standard resistance,

Rx     unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected such that the leads from +C and + P are

connected to one end and those from –C and –P are connected to the other end in the

kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing the galvanometer key.

3. The ‘S’ arm (main dial) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point. [If not some other P/Q

ratio is to be tried].

4. The lowest of the two position indicates the coarse value of the unknown resistance

and the null point is obtained by adjusting the Vernier scale, with the galvanometer

sensitivity knob at the maximum position.

5. The value of unknown resistance is read. [‘S’ Value]

6. Steps 3, 4, 5 are repeated for some other P/Q ratio for the unknown resistance. The

mean value is taken.

7. The above procedure is repeated with another sample.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (Multiplier) | S VALUE COARSE() FINE ( ) | UNKNOWN RESISTANCE Rx () |
| --- | --- | --- | --- | --- |

.

CALCULATION:

RESULT:

EXPT. NO. 3	MEASUREMENT OF CAPACITANCE USING

SCHERING BRIDGE

# AIM:

To measure the unknown capacitance using Schering bridge.

# OBJECTIVE:

To measure the unknown capacitance and to study about dissipation factor.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 | Schering Bridge kit Unknown capacitance Multimeter Connecting Wires CRO |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

1.  Unknown capacitance, Cx = (R1/R2) C3,

Where   C3 is known capacitance in µF

R2 is Non-Inductive Variable Resistor

2. Dissipation factor, D = ω R1 C1 =  ω Rx Cx

PROCEDURE:

- The oscillator AB and the bridge AB terminals are connected.

- From the CD terminals of the bridge, the detector (loudspeaker) is connected through an imbalance amplifier.

- Connections of the bridge arm are made as per the circuit diagram.

- The value of R2 is selected arbitrarily (say1K) and R1 is kept at maximum position.

- The kit is switched on and R1 is decreased until the null point is observed as a dip in the sound from the loudspeaker.

- The capacitor C1 can be varied for fine balance adjustment.

- When the balance condition is reached, the trainer kit is switched OFF and the value R1 is measured using a multimeter.

- The value of unknown capacitance and dissipation factor is calculated.

- The experiment is repeated for various samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R2 () | R1 () | UNKNOWN CAPACITANCE Cx (µF) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 4 	MEASUREMENT OF INDUCTANCE USING

MAXWELLS BRIDGE

# AIM:

To find the unknown inductance and Q factor of a given coil.

# OBJECTIVE:

To find the unknown inductance of the given coil using bridge circuit and to study that Maxwell inductance- capacitance bridge is suitable for the measurement of low Q coils.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Maxwell’s inductance Capacitance Bridge kit Unknown Inductance Multimeter Connecting Wires CRO Bridge oscillator | 1 kHZ |  |  |

CIRCUIT DIAGRAM:

R3

L1

R1

D

C4

R2

R4

# THEORY:

# FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

Thus we have two variables R4 and C4 which appears in one of the two balance equation and hence the two equations are independent, and balance is obtained by varying R4 and C4 alternately.

3. Quality factor, Q = ωL1 / R1 = ωC4R4

PROCEDURE:

- The inductance to be measured is connected between L1 terminal of the kit.

- The bridge oscillator is set for 10Vpp, 1 kHz and connected to the OSC terminal of the kit.

- The detector CRO or headphone is connected to the headphone terminal of the kit.

- The R4 and C4 are adjusted from the highest range (among the 3 range knobs) to obtain the null point in the detector. [ Null point – For increase in R4 and C4 values the point at which the amplitude reduces to a minimum and then increases is null point).

- At the null point the values of R4 and C4 are noted.

- The value of unknown resistance, inductance and quality factor are calculated.

- The experiment is repeated with other samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R4(Ω) | C4 (F) | UNKNOWN INDUCTANCE (H) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 5	CALIBRATION OF SINGLE PHASE ENERGYMETER

AIM:

To calibrate the given single phase energy meter at unity and other power factors

OBJECTIVE:

- To study the working of energy meter.

- To accurately calibrate the meter at unity and other power factor.

- To study the % of error for the given energy meter.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Energy meter Wattmeter Ammeter Voltmeter Stop watch Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

NAME PLATE DETAILS:

| RATED CURRENT |  |
| --- | --- |
| RATED VOLTAGE |  |
| FREQUENCY |  |
| REVOLUTIONS/KWH |  |

THEORY:

FORMULAE USED:

Let x revolution / kwh be the rating.

Now x revolution = 1 kwh

= 1* 3600*1000 watt-sec.

Constant k of energymeter  = 3600 * 103/ x watt-sec

For each load, indicated power Wi  is given as Wi = k/t watts

Where

K= energy meter constant (watt-sec)

t = time for 1 revolution(sec)

% error = Wi – Wa / Wi * 100

Where Wi  is indicated power in watts

Wa is actual power shown by wattmeter in watts

% error can be zero +ve or –ve.

PROCEDURE:

1. Connections are given as shown in the circuit diagram.

2. Supply is switched ON and load is increased in steps, each time noting the readings of

ammeter and wattmeter. Also the actual time taken for 1 revolution of the disc is

measured using stop watch.

3. Step 2 is repeated till rated current of the energy meter is reached.

4. % error is calculated and calibration curve is drawn.

TABULAR COLUMN:

| S.NO | LOAD CURRENT I (Amps) | WATTMETER READING, Wa (W) | INDICATED POWER, Wi (W) | Time taken , t (secs) | % ERROR |
| --- | --- | --- | --- | --- | --- |

CALCULATION:

MODEL GRAPH:

+Ve

% Error

0

Wi

-Ve

NOTE:

From the calibration curve it is possible to predict the error in recording the    energy. So the correction can be applied to the energy meter reading so that correct energy reading can be obtained and used.

RESULT:

EXPT. NO. 6        MEASUREMENT OF THREE PHASE POWER

AND POWER FACTOR

AIM

To conduct a suitable experiment on a 3-phase load connected in star or delta to measure the three phase power and power factor using 2 wattmeter method.

OBJECTIVES

- To study the working of wattmeter

- To accurately measure the 3 phase power

- To accurately measure the power factor

- To study the concept of star connected load and delta connected load

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Ammeter Voltmeter Wattmeter Connecting Wires 3 phase inductive load |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

- Total power, P = W1+W2 (W)

- = Tan-1 3 [(W1-W2) /(W1+W2)]

3.   P.F = Cos

PROCEDURE:

- Connection are made as per the circuit diagram, keeping the inductive load in the

initial position.

- Supply switch is closed and reading of ammeter and wattmeter are noted .If one of the    wattmeter reads negative, then its potential coils (C and V) are interchanged and

reading is taken as negative.

3.  The above procedure is repeated for different values of inductive coil. Care should be

taken that current should not exceed 10A during the experiment.

TABULAR COLUMN:

| S.NO | I | W1 | W2 | POWER, P | P.F Cos |
| --- | --- | --- | --- | --- | --- |
|  | (Amps) | (Watts) | (Watts) | (Watts) |  |

CALCULATION:

RESULT:

EXPT. NO. 7	INSTRUMENTATION AMPLIFIER.

AIM:

To study the working of instrumentation amplifier.

OBJECTIVE:

- To study the characteristic of operational amplifier.

- To study the use of operational amplifier as instrumentation amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Instrumentation Amplifier Kit Dual RPS Multi meter Connecting Wires |  |  |

PIN DIAGRAM:

PIN DIAGRAM OF IC741

CIRCUIT DIAGRAM:

R1 = R2 = R

THEORY:

FORMULAE USED:

- Vo= (V2 – V1) (1 + 2R / Rg)

PROCEDURE:

- Connect a dual RPS to the input terminals V1 and V2 of either channel 1 or channel 2 of instrumentation amplifier module.

- Adjust the RPS for different voltages for V1 and V2 (not more than 10 V).

- Measure the output voltage using a multi meter for every set of V1 and V2.

TABULAR COLUMN:

| V1 (v) | V2 (v) | Vout (v) |
| --- | --- | --- |

RESULT:

EXPT. NO. 8 	STUDY OF DISPLACEMENT TRANSDUCER - LVDT

# AIM

To study the operation of LVDT- Linear Variable Differential Transformer

# OBJECTIVE:

- To study the basic principle of LVDT.

- Study of signal conditioning circuit.

3.         Study of LVDT as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | LVDT kit Multimeter Connecting Wires |  |  |

THEORY:

CIRCUIT DIAGRAM:

Fig.1

Fig.2

PATCHING DIAGRAM:

PROCEDURE:

- The trainer is switched and power supply is checked for +12V and -12Vwith the help of multi meter.

- The trainer is switched off and connections are made as per the patching diagram.

- The LVDT is connected in the kit through connector provided in the circuit.

- The displacement of LVDT is kept at one end by  using screw guage.

- The trainer is switched on and the output DC voltage is measured using multimeter by connecting it across DC output + and – respectively.

- The displacement is varied with the help of screw gauge and the corresponding output voltages are noted down.

- The screw gauge is adjusted for maximum displacement and then stopped.

- A graph between displacement and output voltage is drawn.

TABULAR COLUMN:

| S.NO | DISPLACEMENT ON MICRO-METER (mm) | MULTIMETER READING (Volts) |
| --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 9                          STUDY OF PRESSURE TRANSDUCER

# AIM

To study the operation of a pressure transducer – Strain gauge.

# OBJECTIVES

- To study the basic principle of   Strain gauge.

- Study of Strain gauge as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 | Pressure measurement module Multimeter |  |  |  |

THEORY:

FORMULAE USED:

% Error = [(Gauge pressure – displayed pressure) / displayed pressure] * 100

PROCEDURE:

- Install the pressure cell setup and interface the 9 pin D connector with the pressure measurement module.  Switch ON the module.

- Calibrate the pressure measurement module   by   the following procedure. Open  the air release valve and exhaust the tank inlet air and nullify the output voltage by using zero adjustment pot, then close the air release valve and apply 20 Psi  pressure to the cylinder and adjust the display to 20 Psi by using gain adjustment POT.

- After calibration open the air release valve and exhaust the tank inlet air.

- Close the air release valve and press the pump piston and note down the bridge voltage in mV across T2 and T3, output   voltage   in V across T5 and GND and the displayed pressure in Psi in the indicator.  Repeat the procedure for increasing values of pressure and tabulate the readings.(Pressure should not exceed 20 Psi)

- Plot the error calibration curve.

CIRCUIT DIAGRAM:

TABULAR COLUMN:

| S. No. | Gauge Pressure (Psi) | Displayed Pressure (Psi) | Bridge Voltage (mV) | Output Voltage (V) | %Error |
| --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 10			A/D CONVERTER

AIM

To study the working of successive approximation type Analog to Digital converter.

OBJECTIVE

- To study the conversion of analog i/p voltage to digital o/p voltage.

- To study the operation and characteristics of operational amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | A/D Converter kit RPS Multimeter Connecting Wires |  |  |  |

THEORY:

FORMULAE USED:

Input voltage, Vin = Vref [D7 2-1 + D6 2-2 + D5 2-3 + ………. + D0 2-8]

Where,

Vref = Reference voltage

D7, D6,D5,……D0 = 8 bit digital output

D7  - MSB of digital data

D0 – LSB of digital data

CIRCUIT DIAGRAM:

TABULAR COLUMN:

Reference Voltage –

SW1 =0			SW2 = 1				SW3=1

| Analog input voltage |  | Digital output |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theoretical | Practical | D7 | D6 |  | D5 | D4 |  | D3 | D2 |  | D1 | D0 |

PROCEDURE:

- The channel 3 is selected by using switches SW1, SW2, SW3 by keeping them in 011 position.

- Vref is measured using multi meter across pin 6 and 1 by keeping the pot at maximum position.

- The analog voltage to be converted is set by using the pot and the voltage set is measured by using multi meter.

- The ALE switch SW5 is pressed once to send the data to SAR.

- The SOC switch SW4 is pressed once to obtain the digital data for the given input voltage. The digital output obtained is noted.

- The procedure is repeated from step 2 for various values of analog input voltage.

RESULT:

EXPT. NO. 11			D/A CONVERTER

AIM

To study the working of an 8 bit weighted resistor type Digital to Analog converter.

OBJECTIVE

- To study the conversion of binary voltage to analog o/p voltage

- To study the operation and characteristics of operational amplifier

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | D/A Converter kit RPS Multimeter Connecting Wires |  |  |

THEORY:

FORMULAE USED:

Eo = Vref [(2 Xo – 255) / 256]

Where Xo is the equivalent decimal value of the given digital output.

CIRCUIT DIAGRAM:

PROCEDURE:

- The connections are made as per the circuit diagram.

- Vref is measured across P2 terminal by keeping the pot knob in maximum position.

- The binary input is applied by closing the appropriate switches.

- The analog output across P2 terminal by using multi meter.

- The procedure is repeated for various binary input values.

TABULAR COLUMN:

| S.No. | Digital Input |  | Analog output |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Binary | Decimal equivalent | Theoretical |  | Practical |

RESULT:

EXPT.NO. 12				STUDY OF TRANSIENTS

- RC TRANSIENTS:

AIM

To Observe and plot the transients waveform for a series RC circuit and compute the time constant.

OBJECTIVE

- To study the transient behaviour of the given system

- To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Capacitance Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4τ -1 where τ =RC

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR = Ve- t/RC is observed in the CRO and the time taken by the output to reach 36.8% of its final value (discharging time) is noted.

MODEL GRAPH:

RESULT:

- RL TRANSIENTS:

AIM:

To observe and plot the transient waveform for a series RL circuit and to compute the time constant

OBJECTIVE

1. To study the transient behavior of the given system

2. To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Inductor Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4 τ)-1 where τ =L/R

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR =V ( 1 - e- tR/L) is observed in the CRO and the time taken by the output to reach 63.2% of its final value (charging time) is noted.

MODEL GRAPH:

RESULT:

EXPT. NO. 13	CALIBRATION OF CURRENT TRANSFORMER

AIM

To study the working of current transformer

OBJECTIVE

- To study the current transformation concept

- To study the efficiency of a given current transformer

- To study the loss components in the circuit

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Current Transformer Voltmeter Ammeter Resistive Load |  |  |

THEORY:

FORMULAE USED:

% Ratio error =[( nominal ratio – actual ratio) / actual ratio] x 100

= [ (Kn  - Kact) / Kact ]

Where

Kact  = magnitude of actual primary current / magnitude of actual  secondary current

Kn  = rated primary current / rated secondary current

Inductive reactance in Ω, XL = 2πfL

Where L is inductance of CT secondary in H

Impedance in Ω, Z = √ R2 + XL2

Secondary burden due to load, VA = I2 Z

PRECAUTIONS:

- SPST 1 and SPST 2 should be kept closed before switching ON the supply.

- SPST2 should be opened before applying the load.

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- DPST is closed on the primary side and no load voltage and current are noted.

- SPST 2 is opened and then load is applied in steps. For each load applied, line voltage, primary current, secondary current, and load values are noted down. Also resistance and inductance values of CT secondary are measured using LCR meter.

- The % error and secondary burden due to lad are calculated.

- A graph is drawn between primary current and secondary current is drawn.

TABULAR COLUMN:

| S. No | Line voltage V | Primary current A | Secondary current A | Load W | R Ω | L H | Z Ω | % Error % | Secondary burden due to load VA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO. 14	MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE)

AIM

To measure the iron loss of a given ring specimen

OBJECTIVE

- To study about hysterisis loss

- To study about eddy current loss

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Measurement of iron loss trainer Ring Specimen Multimeter CRO |  |  |

THEORY:

FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

3.  Iron loss = I12 x (R1 – Rw)

Where,

Rw is the winding resistance of the  ring specimen measured using  multimeter.

CIRCUIT DIAGRAM:

PROCEDURE:

- Patch the circuit as per the circuit diagram

- The pot 2(R4) is kept in the maximum position and R3 is selected as 10 Ω initially.

- The trainer kit is switched ON

- Adjust pot 1 (R2) and obtain the null position in the CRO.

- At  the  bridge balance condition  note down the current I1 using multi meter(in mA range selection) connected in series with R3  .

- Switch OFF the trainer kit and note down the value of R2 and winding resistance of ring specimen using multi meter.

- Steps 3 to 6 are repeated for other two settings of R3.

- The entire procedure is repeated for other samples of ring specimens provided in the kit.

TABULAR COLUMN:					R4  = __________ (Ω)

| S.No. | Specimen No. | R2 (Ω) | R3(Ω) | I1 (mA) | RW(Ω) | R1(Ω) | L1(H) | Iron loss (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | 1 |  | 10 |  |  |  |  |  |
| 2. |  |  | 100 |  |  |  |  |  |
| 3. |  |  | 1000 |  |  |  |  |  |
| 4. | 2 |  | 10 |  |  |  |  |  |
| 5. |  |  | 100 |  |  |  |  |  |
| 6. |  |  | 1000 |  |  |  |  |  |
| 7. | 3 |  | 10 |  |  |  |  |  |
| 8. |  |  | 100 |  |  |  |  |  |
| 9. |  |  | 1000 |  |  |  |  |  |

RESULT:

Faculty Name		: Mrs. R. N. Ponnalagu                 	Designation	: Assistant Professor

Department		: E.E.E.				Faculty Code	: EE 06

Subject Name		: M & I Laboratory             	Subject Code	: 131353

Year			: II	                             	Semester	: III

Degree & Branch	: B.E. – E.E.E.			Batch                   : B2

MEASUREMENTS AND INSTRUMENTATION LABORATORY –131353

LIST OF EXPERIMENTS

| S.NO | TITLE | PAGE NO |
| --- | --- | --- |
| 1 | MEASUREMENT OF RESISTANCE USING WHEATSTONE BRIDGE |  |
| 2 | MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE. |  |
| 3 | MEASUREMENT OF CAPACITANCE USING SCHERING BRIDGE. |  |
| 4 | MEASUREMENT OF INDUCTANCE USING MAXWELLS BRIDGE. |  |
| 5 | CALIBRATION OF 1 ENERGYMETER. |  |
| 6 | MEASUREMENT OF 3 PHASE POWER AND POWER FACTOR |  |
| 7 | INSTRUMENTATION AMPLIFIER. |  |
| 8 | STUDY OF DISPLACEMENT TRANSDUCER - LVDT |  |
| 9 | STUDY OF PRESSURE TRANSDUCER |  |
| 10 | A/D CONVERTER |  |
| 11 | D/A CONVERTER |  |
| 12 | STUDY OF TRANSIENTS |  |
| 13 | CALIBRATION OF CURRENT TRANSFORMER |  |
| 14 | MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE) |  |

EXPT. NO. 1	MEASUREMENT OF RESISTANCE USING

WHEATSTONE BRIDGE

# AIM:

To measure the given medium resistance using Wheatstone bridge.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Wheat stone Bridge kit Unknown resistance Multimeter Connecting Wires. |  |  |  |

CIRCUIT DIAGRAM:

# THEORY:

FORMULAE USED:

Unknown Resistance, Rx = (P/Q) S ()

Where   P, Q = Ratio Arms.

S = Variable resistance,

Rx = Unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected between XX points in the bridge kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing both the battery and the galvanometer keys.

3. The S arm (X 1000) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point and  kept at the lowest

value of S. Then the x100, x10, x1 knobs of S are adjusted to get null deflection.

If necessary the sensitivity knob may be adjusted to get appreciable deflection. [If not

possible P/Q ratio is kept at  any other suitable value ie, any one of ratios provided.]

4. The value of unknown resistance is read. (S value)

5. Steps 3 and 4 are repeated for some other P/Q ratio. The mean value is taken.

6. The experiment is repeated with other samples provided.

NOTE: The above experiment may be used for measuring resistance of the samples less than 1 to greater than 10k with lesser sensitivity.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (MULTIPLIER) | S VALUE () | UNKNOWN RESISTANCE RX () |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 2	MEASUREMENT OF RESISTANCE USING KELVIN’S DOUBLE BRIDGE

# AIM:

To measure the given low resistance using Kelvin’s double bridge method.

# OBJECTIVE:

To study the working of bridge under balanced and unbalanced condition and to  study the sensitivity of bridge.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | Kelvin Double bridge kit Unknown resistance Multimeter Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

Rx = (P/Q)  S ohms

Where

P, Q  first set of ratio arms.

p, q   Second set of ratio arms.

S       Standard resistance,

Rx     unknown resistance.

PROCEDURE:

1. The resistance to be measured is connected such that the leads from +C and + P are

connected to one end and those from –C and –P are connected to the other end in the

kit.

2. The P/Q ratio (multiplier) is initially kept at position ‘1’ and the deflection of the

galvanometer is observed by pressing the galvanometer key.

3. The ‘S’ arm (main dial) is adjusted and two positions are identified for which the

deflection of the galvanometer is on either side of the null point. [If not some other P/Q

ratio is to be tried].

4. The lowest of the two position indicates the coarse value of the unknown resistance

and the null point is obtained by adjusting the Vernier scale, with the galvanometer

sensitivity knob at the maximum position.

5. The value of unknown resistance is read. [‘S’ Value]

6. Steps 3, 4, 5 are repeated for some other P/Q ratio for the unknown resistance. The

mean value is taken.

7. The above procedure is repeated with another sample.

TABULAR COLUMN:

| S.NO | SAMPLE | P/Q RATIO (Multiplier) | S VALUE COARSE() FINE ( ) | UNKNOWN RESISTANCE Rx () |
| --- | --- | --- | --- | --- |

.

CALCULATION:

RESULT:

EXPT. NO. 3	MEASUREMENT OF CAPACITANCE USING

SCHERING BRIDGE

# AIM:

To measure the unknown capacitance using Schering bridge.

# OBJECTIVE:

To measure the unknown capacitance and to study about dissipation factor.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 | Schering Bridge kit Unknown capacitance Multimeter Connecting Wires CRO |  |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

1.  Unknown capacitance, Cx = (R1/R2) C3,

Where   C3 is known capacitance in µF

R2 is Non-Inductive Variable Resistor

2. Dissipation factor, D = ω R1 C1 =  ω Rx Cx

PROCEDURE:

- The oscillator AB and the bridge AB terminals are connected.

- From the CD terminals of the bridge, the detector (loudspeaker) is connected through an imbalance amplifier.

- Connections of the bridge arm are made as per the circuit diagram.

- The value of R2 is selected arbitrarily (say1K) and R1 is kept at maximum position.

- The kit is switched on and R1 is decreased until the null point is observed as a dip in the sound from the loudspeaker.

- The capacitor C1 can be varied for fine balance adjustment.

- When the balance condition is reached, the trainer kit is switched OFF and the value R1 is measured using a multimeter.

- The value of unknown capacitance and dissipation factor is calculated.

- The experiment is repeated for various samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R2 () | R1 () | UNKNOWN CAPACITANCE Cx (µF) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 4 	MEASUREMENT OF INDUCTANCE USING

MAXWELLS BRIDGE

# AIM:

To find the unknown inductance and Q factor of a given coil.

# OBJECTIVE:

To find the unknown inductance of the given coil using bridge circuit and to study that Maxwell inductance- capacitance bridge is suitable for the measurement of low Q coils.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Maxwell’s inductance Capacitance Bridge kit Unknown Inductance Multimeter Connecting Wires CRO Bridge oscillator | 1 kHZ |  |  |

CIRCUIT DIAGRAM:

R3

L1

R1

D

C4

R2

R4

# THEORY:

# FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

Thus we have two variables R4 and C4 which appears in one of the two balance equation and hence the two equations are independent, and balance is obtained by varying R4 and C4 alternately.

3. Quality factor, Q = ωL1 / R1 = ωC4R4

PROCEDURE:

- The inductance to be measured is connected between L1 terminal of the kit.

- The bridge oscillator is set for 10Vpp, 1 kHz and connected to the OSC terminal of the kit.

- The detector CRO or headphone is connected to the headphone terminal of the kit.

- The R4 and C4 are adjusted from the highest range (among the 3 range knobs) to obtain the null point in the detector. [ Null point – For increase in R4 and C4 values the point at which the amplitude reduces to a minimum and then increases is null point).

- At the null point the values of R4 and C4 are noted.

- The value of unknown resistance, inductance and quality factor are calculated.

- The experiment is repeated with other samples provided.

TABULAR COLUMN:

| S.NO | SAMPLE | R4(Ω) | C4 (F) | UNKNOWN INDUCTANCE (H) |
| --- | --- | --- | --- | --- |

CALCULATION:

RESULT:

EXPT. NO. 5	CALIBRATION OF SINGLE PHASE ENERGYMETER

AIM:

To calibrate the given single phase energy meter at unity and other power factors

OBJECTIVE:

- To study the working of energy meter.

- To accurately calibrate the meter at unity and other power factor.

- To study the % of error for the given energy meter.

APPARATUS REQUIRED:

| SL.NO | NAME OF THE APPRATUS | RANGE | TYPE | QTY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Energy meter Wattmeter Ammeter Voltmeter Stop watch Connecting Wires |  |  |  |

CIRCUIT DIAGRAM:

NAME PLATE DETAILS:

| RATED CURRENT |  |
| --- | --- |
| RATED VOLTAGE |  |
| FREQUENCY |  |
| REVOLUTIONS/KWH |  |

THEORY:

FORMULAE USED:

Let x revolution / kwh be the rating.

Now x revolution = 1 kwh

= 1* 3600*1000 watt-sec.

Constant k of energymeter  = 3600 * 103/ x watt-sec

For each load, indicated power Wi  is given as Wi = k/t watts

Where

K= energy meter constant (watt-sec)

t = time for 1 revolution(sec)

% error = Wi – Wa / Wi * 100

Where Wi  is indicated power in watts

Wa is actual power shown by wattmeter in watts

% error can be zero +ve or –ve.

PROCEDURE:

1. Connections are given as shown in the circuit diagram.

2. Supply is switched ON and load is increased in steps, each time noting the readings of

ammeter and wattmeter. Also the actual time taken for 1 revolution of the disc is

measured using stop watch.

3. Step 2 is repeated till rated current of the energy meter is reached.

4. % error is calculated and calibration curve is drawn.

TABULAR COLUMN:

| S.NO | LOAD CURRENT I (Amps) | WATTMETER READING, Wa (W) | INDICATED POWER, Wi (W) | Time taken , t (secs) | % ERROR |
| --- | --- | --- | --- | --- | --- |

CALCULATION:

MODEL GRAPH:

+Ve

% Error

0

Wi

-Ve

NOTE:

From the calibration curve it is possible to predict the error in recording the    energy. So the correction can be applied to the energy meter reading so that correct energy reading can be obtained and used.

RESULT:

EXPT. NO. 6        MEASUREMENT OF THREE PHASE POWER

AND POWER FACTOR

AIM

To conduct a suitable experiment on a 3-phase load connected in star or delta to measure the three phase power and power factor using 2 wattmeter method.

OBJECTIVES

- To study the working of wattmeter

- To accurately measure the 3 phase power

- To accurately measure the power factor

- To study the concept of star connected load and delta connected load

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Ammeter Voltmeter Wattmeter Connecting Wires 3 phase inductive load |  |  |

CIRCUIT DIAGRAM:

THEORY:

FORMULA USED:

- Total power, P = W1+W2 (W)

- = Tan-1 3 [(W1-W2) /(W1+W2)]

3.   P.F = Cos

PROCEDURE:

- Connection are made as per the circuit diagram, keeping the inductive load in the

initial position.

- Supply switch is closed and reading of ammeter and wattmeter are noted .If one of the    wattmeter reads negative, then its potential coils (C and V) are interchanged and

reading is taken as negative.

3.  The above procedure is repeated for different values of inductive coil. Care should be

taken that current should not exceed 10A during the experiment.

TABULAR COLUMN:

| S.NO | I | W1 | W2 | POWER, P | P.F Cos |
| --- | --- | --- | --- | --- | --- |
|  | (Amps) | (Watts) | (Watts) | (Watts) |  |

CALCULATION:

RESULT:

EXPT. NO. 7	INSTRUMENTATION AMPLIFIER.

AIM:

To study the working of instrumentation amplifier.

OBJECTIVE:

- To study the characteristic of operational amplifier.

- To study the use of operational amplifier as instrumentation amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Instrumentation Amplifier Kit Dual RPS Multi meter Connecting Wires |  |  |

PIN DIAGRAM:

PIN DIAGRAM OF IC741

CIRCUIT DIAGRAM:

R1 = R2 = R

THEORY:

FORMULAE USED:

- Vo= (V2 – V1) (1 + 2R / Rg)

PROCEDURE:

- Connect a dual RPS to the input terminals V1 and V2 of either channel 1 or channel 2 of instrumentation amplifier module.

- Adjust the RPS for different voltages for V1 and V2 (not more than 10 V).

- Measure the output voltage using a multi meter for every set of V1 and V2.

TABULAR COLUMN:

| V1 (v) | V2 (v) | Vout (v) |
| --- | --- | --- |

RESULT:

EXPT. NO. 8 	STUDY OF DISPLACEMENT TRANSDUCER - LVDT

# AIM

To study the operation of LVDT- Linear Variable Differential Transformer

# OBJECTIVE:

- To study the basic principle of LVDT.

- Study of signal conditioning circuit.

3.         Study of LVDT as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | LVDT kit Multimeter Connecting Wires |  |  |

THEORY:

CIRCUIT DIAGRAM:

Fig.1

Fig.2

PATCHING DIAGRAM:

PROCEDURE:

- The trainer is switched and power supply is checked for +12V and -12Vwith the help of multi meter.

- The trainer is switched off and connections are made as per the patching diagram.

- The LVDT is connected in the kit through connector provided in the circuit.

- The displacement of LVDT is kept at one end by  using screw guage.

- The trainer is switched on and the output DC voltage is measured using multimeter by connecting it across DC output + and – respectively.

- The displacement is varied with the help of screw gauge and the corresponding output voltages are noted down.

- The screw gauge is adjusted for maximum displacement and then stopped.

- A graph between displacement and output voltage is drawn.

TABULAR COLUMN:

| S.NO | DISPLACEMENT ON MICRO-METER (mm) | MULTIMETER READING (Volts) |
| --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 9                          STUDY OF PRESSURE TRANSDUCER

# AIM

To study the operation of a pressure transducer – Strain gauge.

# OBJECTIVES

- To study the basic principle of   Strain gauge.

- Study of Strain gauge as transducer.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 | Pressure measurement module Multimeter |  |  |  |

THEORY:

FORMULAE USED:

% Error = [(Gauge pressure – displayed pressure) / displayed pressure] * 100

PROCEDURE:

- Install the pressure cell setup and interface the 9 pin D connector with the pressure measurement module.  Switch ON the module.

- Calibrate the pressure measurement module   by   the following procedure. Open  the air release valve and exhaust the tank inlet air and nullify the output voltage by using zero adjustment pot, then close the air release valve and apply 20 Psi  pressure to the cylinder and adjust the display to 20 Psi by using gain adjustment POT.

- After calibration open the air release valve and exhaust the tank inlet air.

- Close the air release valve and press the pump piston and note down the bridge voltage in mV across T2 and T3, output   voltage   in V across T5 and GND and the displayed pressure in Psi in the indicator.  Repeat the procedure for increasing values of pressure and tabulate the readings.(Pressure should not exceed 20 Psi)

- Plot the error calibration curve.

CIRCUIT DIAGRAM:

TABULAR COLUMN:

| S. No. | Gauge Pressure (Psi) | Displayed Pressure (Psi) | Bridge Voltage (mV) | Output Voltage (V) | %Error |
| --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO 10			A/D CONVERTER

AIM

To study the working of successive approximation type Analog to Digital converter.

OBJECTIVE

- To study the conversion of analog i/p voltage to digital o/p voltage.

- To study the operation and characteristics of operational amplifier.

APPARATUS REQUIRED:

| S.NO | NAME OF THE APPARATUS | RANGE | TYPE | QUANTITY |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | A/D Converter kit RPS Multimeter Connecting Wires |  |  |  |

THEORY:

FORMULAE USED:

Input voltage, Vin = Vref [D7 2-1 + D6 2-2 + D5 2-3 + ………. + D0 2-8]

Where,

Vref = Reference voltage

D7, D6,D5,……D0 = 8 bit digital output

D7  - MSB of digital data

D0 – LSB of digital data

CIRCUIT DIAGRAM:

TABULAR COLUMN:

Reference Voltage –

SW1 =0			SW2 = 1				SW3=1

| Analog input voltage |  | Digital output |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theoretical | Practical | D7 | D6 |  | D5 | D4 |  | D3 | D2 |  | D1 | D0 |

PROCEDURE:

- The channel 3 is selected by using switches SW1, SW2, SW3 by keeping them in 011 position.

- Vref is measured using multi meter across pin 6 and 1 by keeping the pot at maximum position.

- The analog voltage to be converted is set by using the pot and the voltage set is measured by using multi meter.

- The ALE switch SW5 is pressed once to send the data to SAR.

- The SOC switch SW4 is pressed once to obtain the digital data for the given input voltage. The digital output obtained is noted.

- The procedure is repeated from step 2 for various values of analog input voltage.

RESULT:

EXPT. NO. 11			D/A CONVERTER

AIM

To study the working of an 8 bit weighted resistor type Digital to Analog converter.

OBJECTIVE

- To study the conversion of binary voltage to analog o/p voltage

- To study the operation and characteristics of operational amplifier

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | D/A Converter kit RPS Multimeter Connecting Wires |  |  |

THEORY:

FORMULAE USED:

Eo = Vref [(2 Xo – 255) / 256]

Where Xo is the equivalent decimal value of the given digital output.

CIRCUIT DIAGRAM:

PROCEDURE:

- The connections are made as per the circuit diagram.

- Vref is measured across P2 terminal by keeping the pot knob in maximum position.

- The binary input is applied by closing the appropriate switches.

- The analog output across P2 terminal by using multi meter.

- The procedure is repeated for various binary input values.

TABULAR COLUMN:

| S.No. | Digital Input |  | Analog output |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Binary | Decimal equivalent | Theoretical |  | Practical |

RESULT:

EXPT.NO. 12				STUDY OF TRANSIENTS

- RC TRANSIENTS:

AIM

To Observe and plot the transients waveform for a series RC circuit and compute the time constant.

OBJECTIVE

- To study the transient behaviour of the given system

- To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Capacitance Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4τ -1 where τ =RC

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR = Ve- t/RC is observed in the CRO and the time taken by the output to reach 36.8% of its final value (discharging time) is noted.

MODEL GRAPH:

RESULT:

- RL TRANSIENTS:

AIM:

To observe and plot the transient waveform for a series RL circuit and to compute the time constant

OBJECTIVE

1. To study the transient behavior of the given system

2. To study the effects of transients

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 5 | Resistance Inductor Signal Generator CRO & Probes Bred board & Connecting Wires |  |  |

NOTE

Choose the frequency (f) in signal generator such that f < (4 τ)-1 where τ =L/R

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- Signal generator is adjusted for an input of 10 Vpp square wave and designed frequency and then applied to the circuit.

- The voltage across resistor , VR =V ( 1 - e- tR/L) is observed in the CRO and the time taken by the output to reach 63.2% of its final value (charging time) is noted.

MODEL GRAPH:

RESULT:

EXPT. NO. 13	CALIBRATION OF CURRENT TRANSFORMER

AIM

To study the working of current transformer

OBJECTIVE

- To study the current transformation concept

- To study the efficiency of a given current transformer

- To study the loss components in the circuit

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Current Transformer Voltmeter Ammeter Resistive Load |  |  |

THEORY:

FORMULAE USED:

% Ratio error =[( nominal ratio – actual ratio) / actual ratio] x 100

= [ (Kn  - Kact) / Kact ]

Where

Kact  = magnitude of actual primary current / magnitude of actual  secondary current

Kn  = rated primary current / rated secondary current

Inductive reactance in Ω, XL = 2πfL

Where L is inductance of CT secondary in H

Impedance in Ω, Z = √ R2 + XL2

Secondary burden due to load, VA = I2 Z

PRECAUTIONS:

- SPST 1 and SPST 2 should be kept closed before switching ON the supply.

- SPST2 should be opened before applying the load.

CIRCUIT DIAGRAM:

PROCEDURE:

- Connections are made as per the circuit diagram.

- DPST is closed on the primary side and no load voltage and current are noted.

- SPST 2 is opened and then load is applied in steps. For each load applied, line voltage, primary current, secondary current, and load values are noted down. Also resistance and inductance values of CT secondary are measured using LCR meter.

- The % error and secondary burden due to lad are calculated.

- A graph is drawn between primary current and secondary current is drawn.

TABULAR COLUMN:

| S. No | Line voltage V | Primary current A | Secondary current A | Load W | R Ω | L H | Z Ω | % Error % | Secondary burden due to load VA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MODEL GRAPH:

RESULT:

EXPT. NO. 14	MEASUREMENT OF IRON LOSS (MAXWELL BRIDGE)

AIM

To measure the iron loss of a given ring specimen

OBJECTIVE

- To study about hysterisis loss

- To study about eddy current loss

APPRATUS REQUIRED:

| S.NO | NAME OF THE APPRATUS | RANGE | TYPE QTY |
| --- | --- | --- | --- |
| 1 2 3 4 | Measurement of iron loss trainer Ring Specimen Multimeter CRO |  |  |

THEORY:

FORMULAE USED:

1. Unknown resistance, R1=R2R3 / R4 (Ω)

2. Unknown Inductance, L1=R2R3C4   Henry

3.  Iron loss = I12 x (R1 – Rw)

Where,

Rw is the winding resistance of the  ring specimen measured using  multimeter.

CIRCUIT DIAGRAM:

PROCEDURE:

- Patch the circuit as per the circuit diagram

- The pot 2(R4) is kept in the maximum position and R3 is selected as 10 Ω initially.

- The trainer kit is switched ON

- Adjust pot 1 (R2) and obtain the null position in the CRO.

- At  the  bridge balance condition  note down the current I1 using multi meter(in mA range selection) connected in series with R3  .

- Switch OFF the trainer kit and note down the value of R2 and winding resistance of ring specimen using multi meter.

- Steps 3 to 6 are repeated for other two settings of R3.

- The entire procedure is repeated for other samples of ring specimens provided in the kit.

TABULAR COLUMN:					R4  = __________ (Ω)

| S.No. | Specimen No. | R2 (Ω) | R3(Ω) | I1 (mA) | RW(Ω) | R1(Ω) | L1(H) | Iron loss (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | 1 |  | 10 |  |  |  |  |  |
| 2. |  |  | 100 |  |  |  |  |  |
| 3. |  |  | 1000 |  |  |  |  |  |
| 4. | 2 |  | 10 |  |  |  |  |  |
| 5. |  |  | 100 |  |  |  |  |  |
| 6. |  |  | 1000 |  |  |  |  |  |
| 7. | 3 |  | 10 |  |  |  |  |  |
| 8. |  |  | 100 |  |  |  |  |  |
| 9. |  |  | 1000 |  |  |  |  |  |

RESULT: